# Overpressure Risk Fundamentals & Scenario Definition

# Course Introduction & Overview

# 1. Introduction

# Industrial facilities in the oil & gas, petrochemical, and chemical processing industries operate with equipment that handles fluids under high pressure and temperature conditions. These systems store significant amounts of mechanical and thermodynamic energy, which can become hazardous if pressure exceeds safe operating limits.

# An overpressure event occurs when the pressure inside equipment such as vessels, pipelines, or reactors rises above the maximum allowable working pressure (MAWP). If the pressure is not relieved quickly, the system may experience mechanical failure, rupture, fire, explosion, or toxic release.

# Modern process safety management therefore places strong emphasis on overpressure protection systems, particularly Pressure Safety Valves (PSVs) and flare systems, which are designed to safely relieve excess pressure and prevent catastrophic failures.

# This training course provides participants with a structured understanding of overpressure risk assessment and scenario definition, which form the foundation for designing safe pressure relief and flare systems in industrial facilities.

# 2. Purpose of the Training Program

# The purpose of this course is to provide technical professionals with the knowledge and practical skills required to evaluate and manage overpressure risks in process facilities.

# The course integrates:

- hazard identification

- engineering risk assessment

- consequence modelling

- pressure relief system evaluation

- flare system safety analysis

# Participants will learn how to identify credible overpressure scenarios, evaluate the potential consequences of hazardous releases, and support risk-based design decisions.

# 3. Importance of Overpressure Risk Management

# Overpressure incidents are among the leading causes of major industrial accidents. When pressure containment is lost, the sudden release of energy can cause severe consequences.

# Examples of consequences include:

# • vessel rupture
• large flammable gas releases
• vapor cloud explosions
• jet fires and flash fires
• toxic gas dispersion
• structural damage and personnel injury

# Because of these risks, international engineering standards such as API 520 and API 521 require systematic evaluation of pressure relief scenarios and appropriate design of relief systems.

# Effective overpressure risk management helps organizations:

- prevent catastrophic accidents

- protect personnel and infrastructure

- comply with regulatory requirements

- improve operational reliability

- support safe facility design

# 4. Course Learning Philosophy

# This course follows a structured engineering workflow that reflects real industrial safety studies.

# The learning approach integrates three main elements:

# 1. Hazard Identification

# Understanding how overpressure events can occur in industrial systems.

# 2. Quantitative Risk Evaluation

# Analyzing the likelihood and consequences of hazardous releases.

# 3. Risk-Based Decision Making

# Using modelling results to support engineering design decisions.

# By combining these elements, participants gain practical experience in identifying hazards and evaluating pressure protection strategies.

# 5. Key Concepts in Overpressure Risk

# To understand overpressure risk, several fundamental concepts must be clearly understood.

# 5.1 Pressure Systems in Process Facilities

# Pressure systems include equipment designed to operate under controlled pressure conditions, such as:

- pressure vessels

- reactors

- storage tanks

- pipelines

- compressors

- heat exchangers

# These systems must be protected from conditions that may cause excessive pressure accumulation.

# 5.2 Maximum Allowable Working Pressure (MAWP)

# The maximum allowable working pressure represents the highest pressure at which equipment can safely operate.

# If this pressure is exceeded, structural failure may occur.

# Design codes such as:

- ASME Boiler and Pressure Vessel Code

- API Standards

# define design requirements to ensure safe operation.

# 5.3 Loss of Containment

# Loss of containment occurs when process fluids escape from equipment due to failure or rupture.

# Possible release pathways include:

- small leaks

- pipeline rupture

- vessel rupture

- relief valve discharge

# Loss of containment can result in hazardous events such as:

- fires

- explosions

- toxic exposure

# 5.4 Pressure Relief Systems

# Pressure relief systems are designed to prevent equipment failure by releasing excess pressure.

# Typical relief devices include:

- pressure safety valves (PSVs)

- rupture disks

- relief valves

- emergency vent systems

# These systems protect equipment by allowing controlled release of fluids to a safe disposal system, often through a flare network.

# 6. Role of PSV and Flare Systems in Risk Mitigation

# Pressure Safety Valves (PSVs) are the primary protection mechanism against overpressure in process equipment.

# When system pressure exceeds a predefined limit, the PSV opens and releases fluid to reduce pressure.

# The discharged fluid is typically routed to a flare system, which safely burns off hydrocarbons to prevent accumulation of flammable gases.

# Together, PSVs and flare systems provide:

# • equipment protection
• controlled release of hazardous fluids
• prevention of catastrophic rupture
• safe disposal of emergency releases

# These systems form a critical layer of protection in process safety design.

# 7. Overpressure Scenario Definition

# A key part of pressure system safety analysis is defining credible overpressure scenarios.

# A scenario describes how an abnormal condition could develop and lead to pressure buildup.

# Scenario definition typically includes:

- initiating event

- pressure increase mechanism

- protective systems involved

- possible consequences

# Accurate scenario definition ensures that all relevant hazards are properly evaluated.

# 8. Typical Causes of Overpressure

# Several operating conditions can cause pressure to exceed safe limits.

# Common causes include:

# Blocked Outlet

# If the outlet of a vessel or pipeline becomes blocked, pressure may rapidly increase.

# Fire Exposure

# External fire can heat process equipment and cause vapor expansion.

# Control System Failure

# Instrumentation failure may cause uncontrolled pressure increase.

# Runaway Chemical Reaction

# Exothermic reactions may generate heat and gas rapidly.

# Utility Failure

# Loss of cooling water or power may cause unsafe operating conditions.

# These events must be analyzed to determine relief system requirements.

9. Loss of Containment Pathways

# Once pressure exceeds the containment capacity of equipment, hazardous fluids may escape through different pathways.

# Typical loss of containment pathways include:

# • relief valve discharge
• pipe leak
• flange leak
• gasket failure
• catastrophic rupture

# Each release scenario may result in different consequences depending on fluid properties and operating conditions.

# 10. Hazard Identification for Pressure Systems

# Systematic hazard identification is required to identify potential overpressure risks.

# Common hazard identification techniques include:

- HAZID (Hazard Identification)

- HAZOP (Hazard and Operability Study)

- What-if analysis

- Failure mode analysis

# These methods help engineers identify:

# • potential deviations from normal operation
• initiating events
• possible consequences
• required safety barriers

# 11. Expected Learning Outcomes

# After completing this introductory module, participants will be able to:

# • understand the fundamentals of overpressure risk
• explain the role of pressure relief and flare systems
• identify common causes of overpressure
• define credible accident scenarios
• prepare data for consequence modelling studies
• participate effectively in hazard identification workshops

# These capabilities form the foundation for performing structured process safety analysis in industrial facilities.

# 12. Conclusion

# Overpressure protection is a critical element of process safety management in oil and gas facilities. Proper identification and evaluation of overpressure scenarios are essential for preventing catastrophic accidents.

# Through a combination of theoretical understanding, engineering analysis, and practical exercises, this training program equips participants with the knowledge required to systematically analyze pressure system risks and support safe facility design.

# The skills gained in this course help professionals contribute to risk-based decision making, improved plant safety, and effective pressure relief system management.